Kohltech International Architectural Details

Select Single Hung

System Features	2

Structural Performance	4

Thermal Performance 	6

Section Details: Mullion Details	10

Section Details: Head/Sill, Jamb Details	12

Section Details: Mullion Details	 14

Section Details: Head/Sill, Jamb Details	16

Section Details: Exterior Options	20

Section Details: Head/Sill, Jamb Details	24

Section Details: Interior Options	25

NOTE:

Specifications and technical data are subject to change without notice.

Allow 1/16” (2mm) tolerance on all measurements.

Metric measurements are shown in parenthesis.

For accessories dimensions and applications see the Accessories section of this manual.

For technical assistance you may call 1-800-565-4396 or visit our website:

www.peterkohler.com

Select Single Hung

# System Features

Frame:

- Standard is white extruded polyvinyl chloride (uPVC) profiles.  Optional tan extruded.

- Operator/Fixed Frame depth: 3.25” (83mm)

- Small Picture Window Frame depth: 3.25” (83mm).  This profile is used for shapes.

Sash:

- Standard is white extruded polyvinyl chloride (uPVC) profiles.  Optional tan extruded.

Interior and exterior finish

- Interior finish: Standard high gloss white.  Optional tan.

- Exterior finish: Standard high gloss white.  Optional tan

- Additional colours are also available: waterborne single component color coating system which is chemically bonded to the exterior vinyl surface.

Hardware

- Truth lock and keeper

- John Evans constant force balance

- Stainless pivot bars

Weather Stripping:

.  Two pile weather strip on sash

Jamb Extensions:

- Jamb extensions available for various depths and factory applied.

Insect Screens:

- White aluminum full screen standard. Optional tan to match tan extruded frame and sash.

- Screen mesh: grey fiberglass standard.

- Pull tabs integrated into screen corner

Interior / Exterior Simulated Divided Lites (SDL):

- Interior bar: Extruded vinyl or wood, 3/4” and 1” wide bars.  Standard white.  Optional tan or painted.

- Exterior bar: Extruded vinyl, 3/4” and 1” wide bars.  Standard white.  Optional tan or painted.

- Pattern: Rectangular, custom lite layouts available, contact your Kohltech International representative

Between the Glass Grilles:

- 3/4” Contoured Georgian.  Standard white.  Optional tan, painted any standard color, or spilt finish interior/exterior.

- 1” Contoured Georgian.  Standard white.  Optional painted any standard color.

- 5/8” Flat Colonial.  Standard white.  Optional tan, brass, brushed aluminum, painted.

- Slimline 5/16” Flat Colonial.  Standard white.  Optional tan, pewter, black, gold, mill finish, painted.

Select Single Hung

# System Features

Glazing:

- Glazing method: Insulated glass, hermetically sealed

- Glazing seal: Tape glazed, silicone heel bead

- Glazing type: Clear glass

- Spacer type: Warm edge steel Intercept

- Options: soft coat LoE, argon gas fill, obscure glass, tinted glass, Energlas (LoE/Argon and Stainless Steel spacer), Energlas Plus (triple paned, 2 panes with LoE glass, argon gas filling, and Edgetech warm edge Superspacer)

Accessories:

- Standard: ½” to ¾” return is built into the frame on the interior.

- Optional:  7/8”,1½”; 2”, or 3 ½” brick molding all four sides or with subsill.  All available with J-trim and nailing fin.  2” and 3 ½” brick molds available with decorative backband molding.

- Installation brackets

Select Single Hung

# Structural Performance

| Supreme Single Hung Series Performance Data – CSA A440 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Gateway Size | Air Resistance | Water Resistance | Wind Load Resistance |
| Operating Single Hung | 40" x 63" (1015mm x 1600mm) 43-1/4” x 84” (1100mm x 2134mm) 50” x 72-5/8” (1272mm x 1846mm) | A3 A3 A3 | B7 B4 B5 | C5 C3 C3 |
| Select Single Glider | 63" x 39" (1600mm x 1000mm) | A3 | B4 | C5 |
| Picture Window | 50" x 100" (1267mm x 2540mm) 72” x 84” (1829mm x 2134mm) | FX FX | B7 B7 | C3 C4 |

| Supreme Single Hung Series Performance Data - NAFS |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Gateway Size | Rating |  | Canadian Air Infiltration |
| Operating Single Hung | 40 x 63 43-1/4 x 84 50 x 72-5/8 | R-PG70-H LC-PG40-H R-PG50-H |  | A3 A3 A3 |
| Select Single Glider | 63 x 39 | R-PG40-H |  | A3 |
| Picture Window | 50 x 100 72 x 84 | R-PG50-FW CW-PG60-FW |  | FX FX |

Select Single Hung

# Structural Performance

| CSA A440-00 Performance Class Indicators |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Air Infiltration (Test Pressure = 75 Pa) |  |  | Water Infiltration |  |  |  |  | Wind Load Resistance |  |  |  |  |
| Infiltration (m3/h/m2) | Rating |  | Test Pressure (Pa) |  | Rating |  |  | Test Pressure (Pa) |  |  | Rating |  |
| ≤ 2.97 | A1 |  | 150 |  | B1 |  |  | ± 500 |  |  | C1 |  |
| ≤ 1.65 | A2 |  | 200 |  | B2 |  |  | ± 750 |  |  | C2 |  |
| ≤ 0.55 | A3 |  | 300 |  | B3 |  |  | ± 1200 |  |  | C3 |  |
| ≤ 0.25 | FX |  | 400 |  | B4 |  |  | ± 1600 |  |  | C4 |  |
|  |  |  | 500 |  | B5 |  |  | ± 2000 |  |  | C5 |  |
|  |  |  | 600 |  | B6 |  |  |  |  |  |  |  |
|  |  |  | 700 |  | B7 |  |  |  |  |  |  |  |
| NAFS (AAMA/CSA-A440) Performance Grade Indicators |  |  |  |  |  |  |  |  |  |  |  |  |
| Product Performance Classes |  | Minimum Performance Grade |  | Minimum Design Pressure |  |  | Minimum Structural Test Pressure |  |  | Minimum Water Penetration Test Pressure |  |  |
|  |  | PG |  | psf |  | (Pa) | psf |  | (Pa) | psf |  | (Pa) |
| R – Residential |  | 15 |  | 15 |  | (719) | 2.90 |  | (137) | 2.90 |  | (137) |
| LC – Light Commercial |  | 25 |  | 25 |  | (1198) | 3.75 |  | (180) | 3.75 |  | (180) |
| CW – Commercial |  | 30 |  | 30 |  | (1437) | 4.50 |  | (217) | 4.50 |  | (217) |
| AW – Architectural |  | 40 |  | 40 |  | (1916) | 8.00 |  | (383) | 8.00 |  | (383) |

Select Single Hung

# Thermal Performance

| Select Single Hung Series Average Unit Thermal Performance Data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | U-value Btu/h•ft.2•°F (W/m2•K) | R-value | SHGC | VT | ER Number | Condensation Resistance | Canadian Energy Star Zone | US Energy Star |
| Operating Single Hung |  |  |  |  |  |  |  |  |
| Clear Glass (2012) | 0.48 (2.7) | 2.08 | 0.6 | 0.63 | 14 | 46 | N/A | N/A |
| LoE (2012) | 0.37 (2.1) | 2.7 | 0.55 | 0.63 | 25 | 50 | N/A | N/A |
| LoE/Argon (2012) | 0.34 (1.9) | 2.94 | 0.55 | 0.63 | 29 | 53 | ABC | N/A |
| LoE – LSG (2012) | 0.35 (2.0) | 2.85 | 0.31 | 0.56 | 14 | 51 | N/A | N/A |
| LoE/Argon – LSG (2012) | 0.32 (1.8) | 3.13 | 0.31 | 0.56 | 17 | 54 | A | N/A |
| Energlas (2012) | 0.33 (1.9) | 3.03 | 0.55 | 0.63 | 30 | 56 | ABC | N/A |
| Energlas – LSG (2012) | 0.31 (1.8) | 3.23 | 0.31 | 0.56 | 19 | 57 | A | NC |

| Select Single Hung Series Average Unit Thermal Performance Data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | U-value Btu/h•ft.2•°F (W/m2•K) | R-value | SHGC | VT | ER Number | Condensation Resistance | Canadian Energy Star Zone | US Energy Star |
| Single Hung Glider |  |  |  |  |  |  |  |  |
| Clear Glass (2012) | 0.48 (2.7) | 2.08 | 0.6 | 0.63 | 14 | 45 | N/A | N/A |
| LoE (2012) | 0.37 (2.1) | 2.7 | 0.55 | 0.63 | 25 | 51 | N/A | N/A |
| LoE/Argon (2012) | 0.34 (1.9) | 2.94 | 0.55 | 0.63 | 29 | 53 | ABC | N/A |
| LoE – LSG (2012) | 0.35 (2.0) | 2.85 | 0.31 | 0.55 | 14 | 52 | N/A | N/A |
| LoE/Argon – LSG (2012) | 0.32 (1.8) | 3.13 | 0.31 | 0.55 | 17 | 54 | A | N/A |
| Energlas (2012) | 0.33 (1.9) | 3.03 | 0.55 | 0.63 | 30 | 57 | ABC | N/A |
| Energlas – LSG (2012) | 0.31 (1.8) | 3.23 | 0.31 | 0.55 | 19 | 58 | A | NC |

Select Single Hung

Thermal Performance

| Select Single Hung Series Average Unit Thermal Performance Data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | U-value Btu/h•ft.2•°F (W/m2•K) | R-value | SHGC | VT | ER Number | Condensation Resistance | Canadian Energy Star Zone | US Energy Star |
| Picture Window |  |  |  |  |  |  |  |  |
| Clear Glass (2012) | 0.47 (2.7) | 2.13 | 0.62 | 0.66 | 17 | 47 | N/A | N/A |
| LoE (2012) | 0.35 (2.0) | 2.85 | 0.58 | 0.67 | 30 | 52 | ABC | N/A |
| LoE/Argon (2012) | 0.31 (1.8) | 3.23 | 0.58 | 0.67 | 35 | 55 | ABCD | N |
| LoE – LSG (2012) | 0.33 (1.9) | 3.03 | 0.32 | 0.58 | 17 | 53 | N/A | N/A |
| LoE/Argon – LSG (2012) | 0.29 (1.6) | 3.45 | 0.32 | 0.58 | 22 | 56 | A | N, NC |
| Energlas (2012) | 0.31 (1.8) | 3.23 | 0.58 | 0.67 | 35 | 58 | ABCD | N |
| Energlas – LSG (2012) | 0.29 (1.6) | 3.45 | 0.32 | 0.58 | 22 | 59 | A | N, NC |

Select Single Hung

# Thermal Performance

Canadian Energy Star Climate Zones

| Energy Star Requirements for Windows (2004) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Maximum U-values and Minimum R-Values |  |  |  |  | Minimum Energy Rating (ER) Values (Maximum U-value 2.00 W/m2•K) |  |  |
| Zone | U-value (W/m2•K) |  | U-value (Btu/h•ft.2•°F) | R-Value (ft.2•h•°F/Btu) |  |  |  | Operating Windows | Fixed Windows |
| A | 2.00 |  | 0.35 | 2.9 |  | Or |  | 17 | 27 |
| B | 1.80 |  | 0.32 | 3.2 |  | Or |  | 21 | 31 |
| C | 1.60 |  | 0.28 | 3.6 |  | Or |  | 25 | 35 |
| D | 1.40 |  | 0.25 | 4.0 |  | Or |  | 29 | 39 |

For more information please visit:

http://oee.nrcan.gc.ca/energystar/english/consumers/ratings.cfm

Select Single Hung

# Thermal Performance

United States Energy Star Climate Zones

Select Single Glider

# Section Details: Mullion

Select Single Glider

# Section Details: Mullion

Select Single Glider

# Section Details: Head/Sill, Jamb

Select Single Glider

# Section Details: Head/Sill, Jamb Details, w/ 1- ½” Integral BM

Select Single Hung

# Section Details: Mullion

Select Single Hung

# Section Details: Mullion

Select Single Hung

# Section Details: Head/Sill, Jamb Details

Select Single Hung

# Section Details: Head/Sill, Jamb Details

Select Single Hung

# Section Details: Head/Sill, Jamb Details, w/ 1-1/2” Integral BM

Select Single Hung

# Section Details: w/ 1-1/2” Integral BM, Head/Sill, Jamb Details

Select Single Hung/Glider

# Section Details: Exterior Options

Select Single Hung/Glider

# Section Details: Exterior Options

Select Single Hung/Glider

# Section Details: Exterior Options

Select Single Hung/Glider Fixed

# Section Details: Exterior Options

Select Single Hung/Glider Fixed

# Section Details: w/ 1-1/2” Integral BM, Head/Sill, Jamb Details

Select Single Hung/Glider

# Section Details: Interior Options

Select Single Hung/Glider

# Section Details: Interior Options