Kohltech International Architectural Details

Wood Entrance Doors

System Features	2

Structural Performance	4

Thermal Performance	5

4-5/8” In-swing Single	6

4-5/8” Out-swing Single	7

6-5/8” In-swing Single	8

6-5/8” Out-swing Single	9

4-5/8” In-swing Double Door (or Single with Furniture Sidelite)	10

4-5/8” Out-swing Double Door (or Single with Furniture Sidelite)	11

6-5/8” In-swing Double Door (or Single with Furniture Sidelite)	12

6-5/8” Out-swing Double Door (or Single with Furniture Sidelite)	13

4-5/8” In-swing Garden Door	14

4-5/8” Out-swing Garden Door	15

6-5/8” In-swing Garden Door	16

6-5/8” Out-swing Garden Door	17

4-5/8” In-swing Sidelite	18

4-5/8” Out-swing Sidelite	19

6-5/8” In-swing Sidelite	20

6-5/8” Out-swing Sidelite	21

4-5/8” In-swing S-100	22

4-5/8” Out-swing S-100	23

6-5/8” In-swing S-100	24

6-5/8” Out-swing S-100	25

4-5/8” In-swing Transom	26

4-5/8” Out-swing Transom	27

6-5/8” In-swing Transom	28

6-5/8” Out-swing Transom	29

In-swing Vertical Mullions	30

Out-swing Vertical Mullions	31

In-swing Horizontal Mullions	32

Out-swing Horizontal Mullions	33

In-swing Storm Door Prep on Post	34

Exterior Options	35

In-swing Sill Options	36

Out-swing Sill Options	37

6-5/8” Out-swing Vented Panel	38

6-5/8” Out-swing Vented Panel – Vertical Mullion	39

# System Features

Frame:

- Standard is white extruded polyvinyl chloride (uPVC) profiles.  Optional tan extruded.

- Operator/Fixed Frame depth: 3.25” (83mm)

- Small Picture Window Frame depth: 3.25” (83mm).  This profile is used for shapes.

Sash:

- Standard is white extruded polyvinyl chloride (uPVC) profiles.  Optional tan extruded.

Interior and exterior finish

- Interior finish: Standard high gloss white.  Optional tan.

- Exterior finish: Standard high gloss white.  Optional tan

- Additional colours are also available: waterborne single component color coating system which is chemically bonded to the exterior vinyl surface.

Hardware

- Truth lock and keeper

- John Evans constant force balance

- Stainless pivot bars

Weather Stripping:

- Two pile weather strip on sash

Jamb Extensions:

- Jamb extensions available for various depths and factory applied.

Insect Screens:

- White aluminum full screen standard. Optional tan to match tan extruded frame and sash.

- Screen mesh: grey fiberglass standard.

- Pull tabs integrated into screen corner

Interior / Exterior Simulated Divided Lites (SDL):

- Interior bar: Extruded vinyl or wood, 3/4” and 1” wide bars.  Standard white.  Optional tan or painted.

- Exterior bar: Extruded vinyl, 3/4” and 1” wide bars.  Standard white.  Optional tan or painted.

- Pattern: Rectangular, custom lite layouts available, contact your Kohltech International representative

Between the Glass Grilles:

- 3/4” Contoured Georgian.  Standard white.  Optional tan, painted any standard color, or spilt finish interior/exterior.

- 1” Contoured Georgian.  Standard white.  Optional painted any standard color.

- 5/8” Flat Colonial.  Standard white.  Optional tan, brass, brushed aluminum, painted.

- Slimline 5/16” Flat Colonial.  Standard white.  Optional tan, pewter, black, gold, mill finish, painted.

Glazing:

- Glazing method: Insulated glass, hermetically sealed

- Glazing seal: Tape glazed, silicone heel bead

- Glazing type: Clear glass

- Spacer type: Warm edge steel Intercept

- Options: soft coat LoE, argon gas fill, obscure glass, tinted glass, Energlas (LoE/Argon and Stainless Steel spacer), Energlas Plus (triple paned, 2 panes with LoE glass, argon gas filling, and Edgetech warm edge Superspacer)

Accessories:

- Standard: ½” to ¾” return is built into the frame on the interior.

- Optional:  7/8”,1½”; 2”, or 3 ½” brick molding all four sides or with subsill.  All available with J-trim and nailing fin.  2” and 3 ½” brick molds available with decorative backband molding.

- Installation brackets

NOTE:

Specifications and technical data are subject to change without notice.

Allow 1/16” (2mm) tolerance on all measurements.

Metric measurements are shown in parenthesis.

For accessories dimensions and applications see the Accessories section of this manual.

For technical assistance you may call 1-800-565-4396 or visit our website:

www.peterkohler.com

# Structural Performance

| Estate Double Hung Series Performance Data - NAFS |  |  |  |
| --- | --- | --- | --- |
|  | Gateway Size | Rating | Canadian Air Infiltration |
| Steel Entrance | 37.5 x 82.375 37.5 x 81.625 37.625 x 82.5625 37.625 x 81.8125 | R-PG30-SHD LW R-PG25-SHD LW R-PG35-SHD LW R-PG25-SHD LW | A2 A2 A3 A3 |
| Steel Sidelite | N/A | N/A | N/A |
| Fibreglass Entrance | 37.5 x 82.375 37.5 x 81.625 37.5 x 82.375 37.5 x 81.625 | R-PG40-SHD LW R-PG40-SHD LW R-PG25-SHD LW R-PG25-SHD LW | A2 A2 A2 A2 |
| Fibreglass Sidelite | N/A | N/A | N/A |
| Garden Door | 75 x 82.375 75 x 82.375 75 x 82.375 | R-PG40-SHD LW R-PG20-SHD LW R-PG35-SHD LW | A3 A3 A3 |
| Double Door | 104.375 x 95.5625 104.875 x 95.5625 105.9375 x 96.625 | R-PG15-SHD LW R-PG15-SHD LW R-PG15-SHD LW | A2 A2 A2 |
| Combination Door | 104.375 x 95.5625 104.875 x 95.5625 105.9375 x 96.625 | R-PG15-SHD LW R-PG15-SHD LW R-PG15-SHD LW | A2 A2 A2 |
| S100 Transom | 104.375 x 95.5625 | N/A | N/A |
| S-100 Sidelite | 104.375 x 95.5625 | N/A | N/A |
| Door Picture Window | 47.24x59.06 | N/A | N/A |

# Thermal Performance

With the exception of most of the clear glass options, all Kohltech / Peter Kohler door systems are Energy Star certified. Please check with you nearest Kohltech / Peter Kohler dealer for information on your particular door system.

# 4-5/8” In-swing Single

# 4-5/8” Out-swing Single

# 6-5/8” In-swing Single

# 6-5/8” Out-swing Single

# 4-5/8” In-swing Double Door (or Single with Furniture Sidelite)

# 4-5/8” Out-swing Double Door (or Single with Furniture Sidelite)

# 6-5/8” In-swing Double Door (or Single with Furniture Sidelite)

# 6-5/8” Out-swing Double Door (or Single with Furniture Sidelite)

# 4-5/8” In-swing Garden Door

# 4-5/8” Out-swing Garden Door

# 6-5/8” In-swing Garden Door

# 6-5/8” Out-swing Garden Door

# 4-5/8” In-swing Sidelite

# 4-5/8” Out-swing Sidelite

# 6-5/8” In-swing Sidelite

# 6-5/8” Out-swing Sidelite

# 4-5/8” In-swing S-100

# 4-5/8” Out-swing S-100

# 6-5/8” In-swing S-100

# 6-5/8” Out-swing S-100

# 4-5/8” In-swing Transom

# 4-5/8” Out-swing Transom

# 6-5/8” In-swing Transom

# 6-5/8” Out-swing Transom

# In-swing Vertical Mullions

# Out-swing Vertical Mullions

# In-swing Horizontal Mullions

# Out-swing Horizontal Mullions

# In-swing Storm Door Prep on Post

# Exterior Options

# In-swing Sill Options

# Out-swing Sill Options

# 6-5/8” Out-swing Vented Panel

# 6-5/8” Out-swing Vented Panel – Vertical Mullion